L20 DOORS/SHUTTERS/HATCHES 
495 REVOLVING DOORS:

Manufacturer:

ASSA ABLOY Entrance Systems Limited, Unit 9 Windmill Business Village, Sunbury-on-Thames, Middlesex, TW16 7DY. Tel: 0333 006 3443. Email: architect.uk.aaes@assaabloy.com Web: www.assaabloyentrance.co.uk

Models:
Besam RD-4-300 STRUCTURAL GLASS Automatic Revolving Door System, with four door leaves, curved glazed fixed side screens, glass ceiling / canopy, complete with IP 65 rated in-floor mounted operator and CDC control. System including normal installation to comply with BS7036-0-2014 / BS-EN16005.

Dimensions:

Internal diameter: 1800 / 2100 / 2400 / 2700 / 3000mm

Overall diameter: XXXXmm (Int. Diameter + 52mm Std. or + 154mm for Night Closing Doors)

Opening Width (TW): 1237 / 1449 / 1661 / 1873 / 2085mm.

Internal Height: XXXXmm (Maximum 3000mm)

Glass Ceiling Trim Height: 45mm. Overall Height XXXXmm

(By others: - Prepared opening).

Standard equipment:
System: IN-FLOOR MOUNTED operator and controls, complete with IP 65 rated containment.

Trim finish: Besam powdercoat finish RAL ref: To be confirmed or Premium Stainless Steel cladding: FINISH TO BE CONFIRMED

STRUCTURAL BOLTED GLASS Door Leaves: Rigid turnstile.

Glass type – Door Leaves: 12mm / 15mm Clear toughened glass (Int. Hgt. >2600mm? then 15mm).

Door Seal: Slender aluminium profile perimeter trim and replaceable horse hair brush to three edges.

STRUCTURAL GLASS Outer Walls: Silicone bonded, with slender aluminium profiles to trim and joints.

Glass type – Outer Walls: 16.76mm 1B1 Clear laminated.

STRUCTURAL GLASS Ceiling / Canopy: Silicone bonded, with slender aluminium profiles to trim and joints.

Glass type - Ceiling / Canopy: 16.76mm 1B1 Clear laminated

Lighting: Imbedded LED glass ceiling 24V, 20 – 60W - OPTIONAL or NONE

Glass Ceiling Console: Concealed Sensor Housing or Standard 45mm Trim.

Control Console; Flush Mount full height Control Post to inner leading mullion or None.
Four Position Programme Selector: Standard SURFACE or FLUSH mounted (Optional Control Post).

Ground Ring; Circular Stainless steel 30 x 30 section formed to suit internal diameter and cross braced to operator cement box. Ring supplied with height and level adjustable fixing brackets.

Matting; EMCO 517G Black Rubber or 517R Black Carpet or NONE

Activation and control system:

CDC control unit with plug-in connections and switched mode power supply 100 – 230V.
Activation: PIR Motion Sensor Surface Mounted to the ceiling perimeter trim or Flush Mounted within Concealed Sensor Housing.

Safety device(s):

PSPE Active soft rubber safety edges to: Outer wall leading mullion and door leaf bottom rail.

ESPE Presence Sensors to: each door leaf leading face

Additional ESPE Presence Sensors for Vulnerable Users to: Outer wall leading mullion or None.

Emergency Stop, on the inner leading mullion: Standard SURFACE or FLUSH mounted (Optional Control Post).

Disabled Slow Speed Switches: Standard SURFACE or FLUSH mounted (Optional Control Post) or None.

Locking mechanism:

Face fixed dead lock to one leaf or Face fixed dead locks to two leaves or Electro-Mechanical lock to the turnstile or Night Closing Doors with face fixed dead locks.

Maintenance:

Warranty with bi annual service visits for optimum performance and longevity

1 YEAR / 2 YEAR / 3 YEAR / 4 YEAR / 5 YEAR / 10 YEAR

Please consult ASSA ABLOY Entrance Systems Limited technical literature for further details and the quotation for project specific notes.