L20 DOORS/SHUTTERS/HATCHES 
495 REVOLVING DOORS:

Manufacturer:

ASSA ABLOY Entrance Systems Limited, Unit 9 Windmill Business Village, Sunbury-on-Thames, Middlesex, TW16 7DY. Tel: 0333 006 3443. Email: architect.uk.aaes@assaabloy.com Web: www.assaabloyentrance.co.uk

Models:
Besam RD-3 FRAME Automatic Revolving Door System, with three door leaves, curved glazed fixed side screens, radially segmented ceiling and semi-circular dustcovers , complete with operator and CDC control. System including normal installation to comply with BS7036-0-2014 / BS-EN16005.

Dimensions:

Internal diameter: 3600mm

Overall diameter: XXXXmm (Int. Diameter + 88mm Std. or + 142mm for NCD)

Opening Width (TW): 1693mm.

Internal Height: XXXXmm (Maximum 2600mm)

Fascia Height: 200mm or 201 – 1250mm Maximum. Overall Height XXXXmm

(By others: - Prepared opening).

Standard equipment:
System: CANOPY MOUNTED operator and controls

Frame finish: SAA or Bronze Anodised or Besam powdercoat finish RAL ref: To be confirmed or Premium / Value Stainless Steel cladding: FINISH TO BE CONFIRMED

FRAME Door Leaves: Rigid Turnstile or Electro Mechanical Breakout – Panic Egress, release force 150N Max.

Glass type – Door Leaves: Standard 6.4mm 2B2 or 9.14mm P2A or 10.28mm P4A or 13mm P5A or 14mm P6B Clear laminated glass.

Door Seal: Soft Edge rubber mounting profile and replaceable horse hair brush to three edges.

FRAME Outer Walls: Standard Curved Glazed side screens or Curved Glazed side screens with class 3 EN1627:2011 burglary resistance.

Glass type – Outer Walls: Standard 8.76mm 1B1 or 9.14mm P2A or 10.28mm P4A or 13mm P5A or 14mm P6B Clear laminated glass.

Four Position Programme Selector: Standard SURFACE or FLUSH or REMOTE mounted.

Ceiling finish: White Melamine or Formica Colour XXXXX

Dustcover finish: White Melamine or Formica Colour XXXXX

Lighting: Twelve No. 12VAC/DC LED spotlights

Canopy Cover; 180 or 360 Degree Weather resistant cover draining to water spouts or NONE

Ground Ring; Circular Stainless steel 40 x 40 section formed to suit internal diameter, supplied with height and level adjustable fixing brackets.

Matting; EMCO 517G Black Rubber or 517R Black Carpet or NONE

Activation and control system:

CDC control unit with plug-in connections and switched mode power supply 100 – 230V.
Activation: PIR or Direction Sensing Radar, concealed behind the canopy fascia. 
Park Control: Rotation Brake, secures the door between each activation or NONE.

Entrapment Control: Re-impulse buttons within the closed compartments or NONE.

Safety device(s):

PSPE Active soft rubber safety edges to; Outer wall leading mullion, door leaf bottom rail and leading door stile.

ESPE Presence Sensors to the outer wall leading mullion and door leaf leading face.

Emergency Stop, on the inner leading mullion: Standard SURFACE or FLUSH mounted.

Disabled Slow Speed Switches: Standard SURFACE or FLUSH mounted or None.

Locking mechanism:

Electro-Mechanical lock to the turnstile or Night Closing Doors with Euro Profile hook lock or Night closing Doors with Deadlock and class 3 EN1627:2011 burglary resistance (Min. 340mm Fascia Height).

Maintenance:

Warranty with two service visits per annum for optimum performance and longevity

1 YEAR / 2 YEAR / 3 YEAR / 4 YEAR / 5 YEAR / 10 YEAR

Please consult ASSA ABLOY Entrance Systems Limited technical literature for further details and the quotation for project specific notes.