L20 DOORS/SHUTTERS/HATCHES 
495 REVOLVING DOORS:

Manufacturer:

ASSA ABLOY Entrance Systems Limited, Unit 9 Windmill Business Village, Sunbury-on-Thames, Middlesex, TW16 7DY.

Tel: 0333 006443. Email: rps.uk.besam@assaabloy.com Web: www.assaabloyentrance.co.uk

Models:
Besam RD-4 SLIM Automatic Revolving Door System, with four door leaves, curved glazed fixed side screens, radially segmented ceiling and semi-circular dustcovers , complete with operator and CDC control. System including normal installation to comply with BS7036-0-2014 / BS-EN16005.

Dimensions:

Internal diameter: 1800 / 2100 / 2400 / 2700 / 3000mm

Overall diameter: XXXXmm (Int. Diameter + 88mm Std. or + 142mm for NCD)

Opening Width (TW): 1191 / 1403 / 1615 / 1827 / 2039mm.

Internal Height: XXXXmm (Maximum 3000mm)

Fascia Height: 200mm or 201 – 1250mm Maximum. Overall Height XXXXmm

(By others: - Prepared opening).

Standard equipment:
System: CANOPY MOUNTED operator and controls

Frame finish: SAA or Bronze Anodised or Besam powdercoat finish RAL ref: To be confirmed or Premium Stainless Steel cladding: FINISH TO BE CONFIRMED

FRAME Door Leaves: Rigid Turnstile or Electro Mechanical Breakout – Panic Egress, release force 150N Max.

Glass type – Door Leaves: Standard 6.4mm 2B2 or 9.14mm P2A Clear laminated glass.

Door Seal: Soft Edge rubber mounting profile and replaceable horse hair brush to three edges.

SLIM Outer Walls: Curved glazed side screens, with top glazing beads flush to the ceiling.

Glass type – Outer Walls: Standard 8.76mm 1B1 Clear laminated.

Four Position Programme Selector: Standard SURFACE or FLUSH or REMOTE mounted.

Ceiling finish: White Melamine or Formica Colour XXXXX

Dustcover finish: White Melamine or Formica Colour XXXXX

Lighting: Six No. 12VAC/DC LED spotlights

Canopy Cover; 180 or 360 Degree Weather resistant cover draining to water spouts or NONE

Ground Ring; Circular Stainless steel 40 x 40 section formed to suit internal diameter, supplied with height and level adjustable fixing brackets.

Matting; EMCO 517G Black Rubber or 517R Black Carpet or NONE

Activation and control system:

CDC control unit with plug-in connections and switched mode power supply 100 – 230V.
Activation: PIR or Direction Sensing Radar, concealed behind the canopy fascia. 
Park Control: Rotation Brake, secures the door between each activation or NONE.

Entrapment Control: Re-impulse buttons within the closed compartments or NONE.

Safety device(s):

PSPE Active soft rubber safety edges to; Outer wall leading mullion, door leaf bottom rail and leading door stile.

Additional ESPE Presence Sensors for Vulnerable Users to: each door leaf face and/or leading mullion or None.

Emergency Stop, on the inner leading mullion: Standard SURFACE or FLUSH mounted.

Disabled Slow Speed Switches: Standard SURFACE or FLUSH mounted or None.

Locking mechanism:

Electro-Mechanical lock to the turnstile or SLIM Night Closing Doors with Euro Profile hook lock.

Maintenance:

Warranty with two service visits per annum for optimum performance and longevity

1 YEAR / 2 YEAR / 3 YEAR / 4 YEAR / 5 YEAR / 10 YEAR

Please consult ASSA ABLOY Entrance Systems Limited technical literature for further details and the quotation for project specific notes.