Crawford Panel Sectional door

Product description:

- Overhead sectional door/sectional door with insulated panels filled with CFC-free polyurethan, panel thickness 42 mm, panel height 545 mm

- Surface, waffled steel or aluminium sheet

- Integrated finger pinch protection

- Top, bottom and side seals and seals between door sections

- Standard track system in galvanized steel

- Spring balancing, 20.000 cycles

- Spring break device

- Lock bolt, step-handle,

- Compliance with all operational and safety instructions in EU Directives and the standards of the European Standardization Committee (CEN)

- Standard hardware (head room 485/510 mm to 800 mm)

- Operating system: manual

- Standard door sizes (DLW x DLH) MIN: 2.050 x 2.150 mm

MAX: 8.000 x 6.000 mm

- Wind load (EN 12424)	Class 3 (700 N/m²) up to door 4.250 x 4.500 mm,

Class 2 (450 N/m²) up to door 8.000 x 6.000 mm.

- Thermal transmittance

(EN 12428)

  - W / (m² K) full panel door (steel)

  - W/(m²K) full panel door

1.25 W/(m² K) with window and passdoor

- Water penetration (EN 12425)	Class 3, (Door 4.000 x 4.000 mm)

- Air permeability (EN 12426)	Class 2 (Door 4.000 x 4.000 mm + passdoor)

Dimensions:

x	mm (WxH)

Glazing options:

- Sandwich glazing, double glazed, rectangular, 600 x 290 mm

- Sandwich glazing, double glazed, oval, 590 x 270 mm

- Aluminium frame bar section with, acrylic sheets, 2 x 3 mm, total thickness 27 mm, single sealed

- Aluminium frame bar section with, scratch resistant acrylic glass, 2 x 3 mm, total thickness 27 mm, single sealed

- Aluminium frame bar section with, acrylic glass 2 x 3 mm, total thickness 27 mm, double sealed

- Aluminium frame bar section with, scratch resistant acrylic glass 2 x 3 mm, total thickness 27 mm, double sealed

- Aluminium frame bar section with, single glazing 3 mm acrylic

- Aluminium frame bar section with, scratch resistant single glazing 3 mm acrylic

- Aluminium frame bar section with, single glazing 4 mm hardened glass

- Triple glazing – scratch resistant

- Triple glazing – not scratch resistant

Surface colour:

- Steel panel

Standard: Polyester coating exterior RAL 1021, 3002, 5010, 6005, 7016, 9002, 9006,

9007, 9010, interior RAL 9002

  - Option: exterior painted in RAL and NCS-standard colour, buyer's choice

  - Option: exterior painted according to colour sample

- Aluminium panel

  - Standard: Aluminium colour interior and exterior

  - Option: Polyester coating exterior RAL 5010, 9010, interior polyester transparent

  - Option: exterior painted in RAL and NCS -standard colour buyer's choice

  - Option: exterior painted according to colour sample

Passdoor:

- Standard: opening outwards, door seals, Left or Right hinged, PC lock, doorway height 1850 mm, doorway width 900 mm, overhead door closer, profile of passdoor E6/EV1, glazing like rest of door, passdoor safety switch

- Low threshold: opening outwards, door seals, DIN L or DIN R hinged, PC lock, doorway height 1850 mm, doorway width 900 mm, overhead door closer, profile of passdoor E6/ EV1, glazing like rest of door, passdoor safety switch

- Option: PC lock with knob cylinder

- Option: Nemef panic lock

- Option: Nemef panic lock with panic bolt

Door locking device:

- Option: Second lock bolt

- Option: Lock with profile cylinder lockable on one or both sides

- Option: Door locking switch (essential with electric operation)

Hardware:

- Standard Low Lift (drop clearance 400 mm to 485/510 mm)

- Low Lift (drop clearance 265/310 mm to 485/510 mm)

- High Lift (drop clearance 800 mm to door height + 390 mm) - maximum lift 3.900 mm

- Vertical Lift (drop clearance min. door height + 350 mm)

Operator:

- Option: Chain hoist "T" (Gear ratio 1:4, Door weight max. 250 kg)

- Option: Chain hoist "U" (Gear ratio 1:3, Door weight over 250 kg)

- Option: Electronic operator:

- Operating voltage: 230 V AC, 50/60 HZ (internal supply 3-phase 230 V AC)

- Operating factor: ED = 30%, S3 10 min, non-continuous operation

- (Engine operation 3 min, pause 7 min, direct-inverting)

- Torque: 15-40 Nm, depending on reel type (value of given door shaft)

- Fusing: 10 A

- Brake: non-reversible worm gear

- Speed monitor: electronic monitoring

- Freewheel / clutch: mechanical engagement/disengagement

- Limit switch: electronic via torque (setting "Door open" during programming) Setting "Door closed" in processor board

- Gears: worm gear 1750 RPM yields approx. 24 rotations door shaft

- Shaft: ø35 mm or hexagonal

- Direct drive system

- No special wall reinforcement necessary

- Temperature range: -20°C to 60°C

- Soft stop, soft start

- Connection using "CEE plug" or lockable mains isolator

- Maximum door weight 400 kg

Options:

  - Chain transmission

  - Chain guard

Other drive variants available on request subject to technical inspection

Control units:

- Control unit

Output 0.37 kW, input 1 x 230 V. Including control unit with standard actuator UP-STOP- DOWN. The door closes using dead-man controls.

Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time

Options:

  - Additional buttons, key switch

  - Door/bridge interlock

- Control unit

Output 0.37 kW, input 1 x 230 V. Including control unit with standard actuator UP-STOP- DOWN. The door closes using pulse control incl. self-monitoring safety edge

Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time

Options:

  - Radio remote control

  - "Open" (if open button is used during closing process - door immediately stops and opens completely)

  - "Automatic close" (safety photo cell compulsory requirement)

  - Door closes automatically after set period (4-60 sec)

  - Pull switch for opening function

  - External push-button, key switch

  - Door/bridge interlock

- Control unit

Output 0.37 kW, input 1 x 230 V. Dimensions: 180 x 380 x 120 mm (wxhxd) including control unit with standard actuator UP-STOP-DOWN and pulse control incl. self- monitoring safety edge. Counter/error code display and servicing info display in top of control cabinet. Operating speed 0.25 m/s, safety rating IP 55. If there is a power failure, the disengage device enables manual operation at any time.

Options:

- Radio remote control

- "Open" (if open button is used during closing process - door immediately stops and opens completely)

- "Automatic close" (safety photo cell compulsory requirement) Door closes automatically after set period (4-60 sec)

- Pull switch for OPEN / CLOSE

- External push-button, key switch

- "One-button function" - opens or closes door, depending on actual status (e.g. radio, pull switch)

- additional safety (e.g. photo cell, radar) – door immediately opens completely

- D-Module (D1-D7)

- Photo cell

- Reduced opening height

- Radar

- Magnetic loop

- Warning lights

- Traffic lights

- Interlocking door function

- Door/bridge interlock

D-Module (automation module)

- D1 functions

- red lamps, inclusive, blinking continuously when door is moving

- red blinking light before "Automatic close"

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- D7 - function incl.

- D2 functions

- Magnetic loop, inclusive, with various functions, e.g.

  - "Safety and checkback signal"

  - "Open and checkback signal"

  - "Safety and open"

  - "Open"

- A DIP switch can also be chosen to provide confirmation as additional safety

- D7 - function incl.

- D3 - functions

- Interlocking door function or mutual interlocking between two doors

- Door/bridge interlocking

- Incl. 24 V AC max. 400 mA for additional units

- Relay for "Door open" or "Door closed"

- D4 functions

- Combination of D1 and D2

- red lamps (inclusive) blinking continuously when door is moving

- red blinking light (inclusive) before "Automatic close"

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- A DIP switch can also be chosen to provide confirmation as additional safety

- D7 - function incl.

- magnetic loop (inclusive) with various functions, e.g.

  - "Safety and checkback signal"

  - "Open and checkback signal"

  - "Safety and open"

  - "Open"

- D5 functions

- Directional sensor traffic light (including red/green)

- Control of oncoming traffic in the doorway

- using "Open interior" or "Open exterior" to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which "Open command" is given first

- D3 - functions

- D6 functions

- Directional sensor traffic light with magnetic loop activation (inclusive), of "Open exterior"

- Control of oncoming traffic in the doorway

- using "Open interior" or "Open exterior" to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which "Open command" is given first

- D3 - functions

- D7 functions

- Interlocking door function or mutual interlocking between two doors

- Door/bridge interlocking

- Incl. 24 V AC max. 400 mA for additional units

- Relay for "Door open" or "Door closed"

Other supplementary equipment

- Anti-corrosive hardware

  - Anti-corrosive package Type "C"

    - Stainless steel hinges and roller brackets

    - Stainless steel screws

    - Stainless steel roller shafts and door cables

    - Aluminium top and bottom profile

    - Powder coated corner bracket (RAL 9002)

  - Anti- corrosive package Type "A"

    - Stainless steel hinges and roller brackets

    - Stainless steel screws

    - Stainless steel roller shaft and door cables

    - Aluminium top and bottom profile

    - Powder coated corner bracket (RAL 9002)

    - Powder coated end caps, trusses, tracks, hangers, connection beams, safety pull bracket (RAL 9002)

  - Anti- corrosive package Type "S"

    - Safety pull cables, stainless steel

  - Anti- corrosive package Type "Z"

    - Electro-galvanized springs (max. 70.000 cycles)

- Cable break device

- Increased cycle spring (20.000 – 100.000)

- Ventilator "P" (free area approx. 170 cm²)

Special models

- Side fixed sections with and without passdoor similar appearance sectional panel door (subject to technical inspection) available on request!!